22MAT220 - Mathematics for Computing 3

Analog computing report

SHM oscillation

D14-Team Members:

Ravishanmugam K – CB.SC.U4AIE24347

Sharvesh Sivagnanam – CB.SC.U4AIE24355

Sri Harini MP – CB.SC.U4AIE24358

Vignes V M –CB.SC.U4AIE24359

Objective:

Goal of the our Analog computing project to understand the working model of the analog computing with the Operational Amplifier to solve the Ordinary Differential Equation , on the basis of Simple Harmonic Motion equation of Oscillations of an body.Study of ADALM m2k device gives us the basic level analog computing , with the OP-AMP and circuit components to solve and learn the ODE .

Operational Amplifier:

The 741 op-amp is a general-purpose integrated circuit used for amplification, filtering, and signal processing. It provides high voltage gain, high input impedance, and low output impedance, making it ideal for analog applications such as amplifiers and signal conditioners. It is internally compensated for stability and usually operates with a dual power supply of ±15V.

The μA741 comes in an 8-pin package, where Pin 1 and 5 are for offset null adjustment, Pin 2 is the inverting input, Pin 3 is the non-inverting input, Pin 4 is the negative supply (V−), Pin 6 is the output, Pin 7 is the positive supply (V+), and Pin 8 is unused. It’s widely used in amplifiers, filters, oscillators, and comparator circuits due to its reliability and ease of use.

ADALM M2K:

The ADALM2000 (Active Learning Module M2K) is a compact, USB-powered, multifunctional data acquisition and signal analysis device developed by Analog Devices. It is designed for students, hobbyists, and engineers to learn and experiment with analog and digital electronics. The M2K acts as a portable lab, replacing bulky instruments like oscilloscopes, waveform generators, and power supplies, making it ideal for hands-on circuit learning and testing through the Scopy software interface

The ADALM2000 provides multiple I/O functions through its 12-pin connector, including two analog input channels (oscilloscope), two analog output channels (waveform generator), digital I/O pins, power supplies (±5V, +3.3V), and a ground reference. These features allow users to measure, generate, and control both analog and digital signals in real time. With its wide functionality and portability, the M2K is widely used in educational labs and prototyping environments for testing circuits, debugging embedded systems, and understanding signal behavior interactively.

Inverting Amplifier:

An inverting amplifier is a basic op-amp configuration where the input signal is applied to the inverting terminal (−) through a resistor, while the non-inverting terminal (+) is grounded. This setup produces an output signal that is 180° out of phase with the input, meaning it inverts the signal’s polarity. The amplification is controlled by the ratio of two resistors, providing precise and stable voltage gain.

In this circuit, the input resistor (Rin) connects the input signal to the inverting input, and the feedback resistor (Rf) connects the output back to the inverting input. The voltage gain of the inverting amplifier is given by Av = −(Rf / Rin), where the negative sign indicates the phase inversion. The output voltage (Vout) is therefore Vout = −(R2 / R1) × Vin. This configuration is commonly used in audio mixers, signal processing circuits, and control systems where accurate and inverted signal amplification is required

Integrator:

An integrator circuit is an op-amp configuration that performs the mathematical operation of integration, producing an output voltage proportional to the time integral of the input signal. It is mainly used to convert a square wave input into a triangular wave or to compute the area under a signal curve. The integrator is essential in analog computers, waveform generators, and signal-processing systems for converting one form of signal into another.

In this circuit, a resistor (R) is connected to the inverting input (−) and a capacitor (C) is placed in the feedback path between the output and the inverting input. The non-inverting input (+) is grounded. The output voltage is given by Vout = −(1/R1C)f ∫ Vin dt, where the negative sign indicates a 180° phase shift between the input and output. This means the output decreases or increases linearly depending on the input polarity, making it ideal for generating ramp signals and performing analog integration in control and signal applications.

SHM Circuit:

a = − ω2x

,is the defining equation of Simple Harmonic Motion (SHM), where:

- a   is acceleration,

- x  is displacement,

- ω  is the angular frequency.

Since acceleration is the second derivative of displacement with respect to time, we can write:

a = d2xdt2

Substituting this into the SHM relation gives:

d2xdt2 = −ω2x

Circuit Diagram :

1.Power Connection for the Circuit :

- Power the Op-Amp pin number 7 for V+ (Red wire), 4 for v-(White wire) with the Adalm m2k and set the voltage to 5v,-5V . Proceed the same for all op-amp used in the Circuit.

- Connect to ground Rail to all the Pin 3 of Op-amp as the circuit as been formatted for the SHM oscillations.

2.Placement of the Resistor and Capacitors in the circuit:

- Place the 10K Ω Resistor for the first two op-amp in pin 2.For the 1st Op-Amp alone place one pin to the Scope(+1, Orange wire) of the Adalm M2k .for the 2nd op-amp place it in the output(Pin 6) of 1st Op-Amp to the Pin 2 of 2nd Op-Amp

- Place the feedback resistor used in the first two op-amps as 1MΩ.3rd Op-Amp feedback resistor is 10KΩ.

- Place the capacitor 0.1µF in Parallel with the Feedback Resistors. For the first two Op-Amp.

- Place 10KΩ in first two Op-Amp's Pin 6 to the Ground pin.Place 1KΩ in last op-amp's Pin6 to the Ground.

3.Connection Establishment :

- Connect the feedback resistors of all op-amps to the Pin2 and Pin6 of the Repective Op-Amp.

- Similalry connect the Pin3 of the all op-amp to the ground .

- Join the Pin 6 of 1st op-amp to the input pin 2 of 2nd op-amp., do the same for the Third op-amp wrt to the second op-amp .At last connect the ouput pin of last op-amp to the input pin of the first op-amp.

4.Initial Condition setting :

- Place an constant +3v supply to the output of the First Op-Amp Pin 6.

- Place an constant +4v supply to the output of the second Op-Amp Pin 6.

5.ADALM M2K Setting:

- Set the input of the signal generator from the W1(yellow wire) to +3v supply . And w2(yellow with white wire ) to +4v.

- Now give the power supply of the Op-Amp by giving the +5v,-5v in Power supply tab.

- Now connect the oscilloscope of +1(Orange wire) to the first Op-Amp input and the +2(Blue wire) to the first integrator ouput .

6.Final Computation:

- After powering the intial condition , wait for 5 seconds  and remove the wire.

- Go to oscilloscope to get the output of the SHM with a as the Channel-1(Orange line  and v in the channel 2 (Voilet line).

- The final Expected output is to be sinosudal wave in orange, and output voilet line shoule phased by 900.

Hardware Assembly:

Results: